# Chapter 2: Virtualization Concepts

## Server virtualization

Kernel-based Virtual Machine (KVM) is an open source virtualization technology built into Linux. It provides hardware assist to the virtualization software, using built-in CPU virtualization technology to reduce virtualization overheads (cache, I/O, memory) and improving security.

QEMU is a hosted virtual machine emulator that provides a set of different hardware and device models for the guest machine. For the host, QEMU appears as a regular process scheduled by the standard Linux scheduler, with its own process memory. In the process, QEMU allocates a memory region that the guest sees as physical and executes the virtual machine’s CPU instructions.

With KVM, QEMU can just create a virtual machine with virtual CPUs (vCPUs) that the processor is aware of and runs native-speed instructions. When a special instruction is reached by KVM, like the ones that interacts with the devices or to special memory regions, vCPU pauses and informs QEMU of the cause of pause, allowing the hypervisor to react to that event.

LibVirt is an open source toolkit to manage virtualization platforms. Libvirt is collection of different software that allows you to manage virtual machines and other virtualization functionality, such as storage and network interface management (see Figure 2.1). LibVirt is proposing to define virtual components in a XML-formatted configurations, that are able to be translated into the QEMU command line.

Figure 2.1	LibVert Compute Node

## Inter Process Communication

Inter process communication (IPC) is a mechanism which allows processes to communicate with each other and synchronize their actions. The communication between these processes can be considered as a method of cooperation between them.

IPC is used in network virtualization in order to be able to exchange data between different distributed processes of a same application (for example, Virtio frontend and backend, Contrail vrouter agent and dataplane, etc.) or between processes of distinct applications (e.g., contrail vrouter and QEMU Virtio, Virtio and VFIO, and so on).

Two different modes of communication are used for IPC:

- Shared Memory: processes are reading and writing information into shared memory region.

- Message Passing: processes are establishing a communication link which will be used to exchange messages.

### Shared Memory

The following scenario is used when shared memory is used for IPC:

- First, a shared memory area is defined (shmget) with a key identifier known by processes involved in the communication.

- Second, processes are attaching (shmat) to the shared memory and are retrieving a memory pointer.

- Then, processes are reading or writing information in the shared memory using the shared memory pointer (read/write operation).

- Next, processes are detaching from the shared memory (shmdt)

- Last, the shared memory area is freed (shmctl)

The following system calls are used in shared memory IPC:

- shmget: create the shared memory segment or use an already created shared memory segment.

- shmat: attach the process to the already created shared memory segment.

- shmdt: detach the process from the already attached shared memory segment.

- shmctl: control operations on the shared memory segment (set permissions, collect information).

### Message passing

Several message passing methods are available to exchange data information between processes:

- eventfd: is a system call that creates an eventfd object (64-bit integer). It can be used as an event wait/notify mechanism by user-space applications, and by the kernel to notify user-space applications of events.

- pipe (and named pipe) is a unidirectional data channel. Data written to the write-end of the pipe is buffered by the operating system until it is read from the read-end of the pipe.

- Unix Domain Socket: domain sockets use the file system as their address space. Processes reference a domain socket as an inode, and multiple processes can communicate using a same socket. The server of the communication binds a Unix socket to a path in the file system, so a client can connect to it using that path.

There are some other mechanisms that can be used by processes to exchange messages (shared file, message queues, network sockets, and signals system calls) but are not described in this document.

## Network Device Architecture and concepts

### Control and Data paths

Two different flows are used by a network application using a NIC device (Figure 2.2):

- Control: manages configuration changes (activation/deactivation) and capability negotiation (speed, duplex, buffer size) between the NIC and network application for establishing and terminating the data path on which data packets will be transferred.

- Data: performs data packets transfer between NIC and network application. Packets are transferred from NIC internal buffer to a host memory area which is reachable by the network application.

Figure 2.2	Two different flows using a NIC device

Each flow is using a well-defined path:

- control path

- data path

### Event Versus Polling-based Packet Processing

The Linux network stack (Figure 2.3) is using an event-based packet processing method. In such a method every incoming packet hitting the NIC:

- is copied in host memory via DMA

- then the NIC generates an interrupt

- then a Kernel module is placing the packet into a socket buffer

- application runs a read system call

For every egress packet generated by the network application:

- application performs a write call on the socket in order to copy the generated packet from the applications user space to a socket buffer

- kernel device driver invokes the NIC DMA engine to transmit the frame onto the wire.

- once transmission is complete, the NIC raises an interrupt to signal transmit completion in order to get socket buffer memory freed.

This method is not efficient when packets are hitting the NIC at a high packet rate. Lots of interrupts are generated, creating lots of context switching (kernel to user and vice-versa).

| Event based packet processing | polling based packet processing |
| --- | --- |

Figure 2.3	Event-based Versus Polling-based

Polling based packet processing is an alternate method (it is used by DPDK). All incoming packets are copied transparently (without generating any interrupt) by the NIC into a specific host memory area region (predefined by the application). At a regular pacing, the network application is reading (polling) packets stored into this memory area.

In the opposing direction, the network application is writing packets into the shared memory area region. A DMA transfer is triggered to copy the packet from the host memory to the NIC card buffers.

No interrupt is used with this method, but it requires the network application to check at a regular interval whether a new packet has hit the NIC. This method is well suited for high rate packet processing: If packets are arriving at a slow rate this algorithm is less efficient as the event-based method.

## Network Devices Virtualization

Like CPU virtualization, two kinds of methods are used to virtualize network devices:

- Software-based emulation

- Hardware-assisted emulation

Software based emulation is widely supported but can suffer of poor performance. Hardware assisted emulation provides good performance thanks to its hardware acceleration but requires hardware that supports some specific features.

### Software-Based Emulation.

Two solutions are proposed for device virtualization with software:

- Traditional Device Emulation (binary translation): the guest device drivers are not aware of the virtualization environment. During runtime, the Virtual Machine Manager (VMM), usually QEMU/KVM, will trap all the IO and Memory-mapped I/O (MMIO) accesses and emulate the device behavior (trap and emulate mechanism).
The Virtual Machine Manager (VMM) emulates the I/O device to ensure compatibility and then processes I/O operations before passing them on to the physical device (which may be different). Lots of VMEXIT (context switching) are generated with this method. It provides poor performance.

- Paravirtualized Device Emulation (virtio): the guest device drivers are aware of the virtualization environment. This solution uses a front-end driver in the guest that works in concert with a back-end driver in the VMM. These drivers are optimized for sharing and have the benefit of not needing to emulate an entire device. The back-end driver communicates with the physical device. Performance is much better than with traditional device emulation.

Software emulated devices can be completely virtual with no physical counterpart or physical ones exposing a compatible interface.

### Hardware-assisted Emulation.

Two solutions are proposed for device virtualization assisted with hardware:

- Direct Assignment: allows a VM to directly access a network device. Thus the guest device drivers can directly access the device configuration space to launch a DMA operation in a safe manner for example, via IOMMU.

- Drawbacks:

- direct assignment has limited scalability. A physical device can only be assigned to one single VM.

- IOMMU must be supported by the host CPU (Intel VT-d or AMD-Vi feature).

- SR-IOV: with SR-IOV, each physical device (physical function) can appear as multiple virtual ones (aka virtual function). Each virtual function can be directly assigned to one VM, and this direct assignment is using the vt-d/IOMMU feature.

- Drawbacks:

- IOMMU must be supported by the host CPU (Intel VT-d or AMD-Vi feature).

- SR-IOV must be supported by the NIC device (but also by the BIOS, the host OS and the guest VM).

## Emulated Network Devices

The following two emulated network devices are provided with QEMU/KVM:

- e1000 device: emulates an Intel E1000 network adapter (Intel 82540EM, 82573L, 82544GC).

- rtl8139 device: emulates a Realtek 8139 network adapter.

## Paravirtualized network device

Virtio is an open specification for virtual machines' data I/O communication, offering a straightforward, efficient, standard and extensible mechanism for virtual devices, rather than boutique per-environment or per-OS mechanisms. It uses the fact that the guest can share memory with the host for I/O to implement that.

Virtio was developed as a standardized open interface for virtual machines (VMs) to access simplified devices such as block devices and network adaptors.

### Virtio frontend and backend

Virtio interface is made of backend component and a frontend component:

- The frontend component is the guest side of the Virtio interface

- The backend component is the host side of the Virtio interface

Figure 2.4	Virtio Frontend and Backend

### Virtio transport protocol

The Virtio network driver is the Virtio frontend component exposed into the guest VM. The virtio network device is the Virtio backend component exposed by the hypervisor. The Virtual Network frontend and backends are interconnected with a transport protocol (usually PCI/PCIe). See Figure 2.5.

The Virtio drivers must be able to allocate memory regions that both the hypervisor and the devices can access for reading and writing, via memory sharing. Two different domains have to be considered for a network device:

- Virtio device initialization, activation or shutdown (control plane)

- Network packets transfer through the Virtio device (data plane)

Figure 2.5	Virtio Transport

The control plane is used for capability exchange negotiation between the host and guest, both for establishing and terminating the data plane. The data plane is used for transferring the actual packets between the host and guest.

Virtqueues are the mechanism for bulk data transport on Virtio devices. They are composed of:

- guest-allocated buffers that the host interacts with (read/write packets)

- descriptor rings

Virqueues are controlled with I/O Registers notification messages:

- Available Buffer Notification: Virtio driver notifies there are buffers that are ready to be processed by the device.

- Used Buffer Notification: Virtio device notifies it has finished processing some buffers.

### Virtio device network backend

The network backend interacts with the emulated NIC and is exposed on the host side. Usually the network backend is a tap device but other backends are proposed with Virtio (SLIRP, VDE, Socket).

Tap devices are virtual point-to-point network devices that the user’s applications can use to exchange L2 packets. Tap devices are requiring the tun kernel module to be loaded. Tun kernel modules create a kind of device in the /dev/net system directory tree (/dev/net/tun).

Each new tap device has a name in the /dev/net/tree filesystem.

### Virtio Net Backend Drawbacks

The usual transport backend used by the Virtio net device presents some inefficiencies:

- syscall and data copy are required for each packet to send or receive through the tap interface (no bulk transfer mode).

- virtio driver (front end) notifies there are one available packet for the virtio device (backend) with an interrupt messages (IOCTL)

- each interrupt message stops vCPU execution and generate a context switch (vmexit). Then the host processes the available packet and resume (vmexit) the VM execution using a syscall.

Each time a packet is sent, the VM stops to get the available packet processed.

Figure 2.5	Virtio Transport

Hypervisor is involved in both Virtio control plane and data plane.

### vhost protocol

The vhost protocol was designed in order to address the Virtio device transport backend limitations. It’s a message-based protocol which allows the hypervisor to offload the data plane to a handler. The handler is a component that manages Virtio data forwarding. The host hypervisor no longer processes packets.

The data plane is fully offloaded to the handler that reads or writes packets to/from the virtqueues. The vhost handler directly accesses the virtqueues memory region as well as sends and receives notification messages.

The vhost handler is made up of two parts:

- vhost-net

- a kernel driver

- it exposes a character device on /dev/vhost-net

- uses ioctls to exchange vhost messages (vhost protocol control plane),

- uses irqfd and ioeventfd file descriptor to exchange notifications with the guest.

- spawns a vhost worker thread

- vhost worker

- a linux thread named vhost-<pid> (<pid> is the hypervisor process ID)

- handles the I/O events (generated by virtio driver or tap device)

- forwards packets (copy operations)

A tap device is still used to communicate the guest instance with the host, but the Virtio dataplane is managed by the vhost handler and is no more processed by the hypervisor (see Figure 2.6). Guest instances are not stopped anymore (context switch with a VMEXIT) at each VirtIO packet transfer. New Virtio vhost-net packet processing backend is completely transparent to the guest who still uses the standard Virtio interface.

Figure 2.6	Virtio Vhost Handler

## Physical Network Device Direct I/O Assignment

KVM guests usually have access to software-based emulated NIC devices (either para-virtualized devices with Virtio or traditional emulated devices). On host machines that have Intel VT-d or AMD IOMMU hardware support, another option is possible – PCI devices may be assigned directly to the guest, allowing the device to be used with minimal performance overhead. Assigned devices are physical devices that are exposed to the VM. This method is also known as passthrough.

The VT-d or AMD IOMMU extensions must be enabled in BIOS in order to be able to conduct device direct assignment. Two methods are supported:

- PCI passthrough: PCI devices on the host system are directly attached to virtual machines, providing guests with exclusive access to PCI devices for a range of tasks. This enables PCI devices to appear and behave as if they were physically attached to the guest virtual machine.

- VFIO device assignment: VFIO improves on previous PCI device assignment architecture by moving device assignment out of the KVM hypervisor and enforcing device isolation at the kernel level.

With VFIO the physical device is exposed to the host user space memory and is made visible from the guest VM it has been assigned.

Figure 2.6	Physical Network Device Direct I/O Assignment

## SR-IOV

The Single Root I/O Virtualization (SR-IOV) specification is defined by the PCI-SIG (PCI Special Interest Group). This is a PCI Express (PCI-e) that extends a single physical PCI function to share its PCI resources as separate virtual functions (VFs).

The physical function contains the SR-IOV capability structure and manages the SR-IOV functionality (it can be used to configure and control a PCIe device).

A single physical port (root port) presents multiple, separate virtual devices as unique PCI device functions (up to 256 virtual functions – depends on device capabilities).

Each virtual device may have its own unique PCI configuration space, memory-mapped registers, and individual MSI-based interrupts. Unlike a physical function, a virtual function can only configure its own behavior. Each virtual function can be directly connected to a VM via PCI device assignment (passthrough mode).

SR-IOV improves network device performance for each virtual machine as it can share a single physical device between several virtual machines using the device direct I/O assignment method.

Figure 2.7	Single Root I/O Virtualization

With SR-IOV, each VM has direct access to the physical network using the assigned virtual function interface allocated to it. They can communicate altogether using the Virtual Ethernet Bridge provided by the NIC card. A virtual switch can also use SRIOV to get access to the physical network. A VM using a SR-IOV assigned virtual function device has direct access to the physical network and is not connected to any intermediate virtual network switch or router (see Figure 2.8).

Figure 2.8	Single Root I/O Virtualization

The following command can be used to check whether SR-IOV is supported or not on a physical NIC card:

$ lspci -s <NIC_BDF> -vvv | grep -i "Single Root I/O Virtualization"

## Virtio, SR-IOV, and SDN

Virtio brings lots of flexibility by offering a standardized driver which is fully independent of the hardware used on the physical platform hosting VM instances.

When Virtio connectivity is used VM can be easily migrated from one host to another using a live migration feature. When SR-IOV is used, this live migration is not an easy task and is not really possible to achieve.

Indeed, network drivers used by a VM depends on used hardware on the bare metal node which are hosting them. In order to make the VM migration from one bare metal node to another, both nodes must at least use the same hardware NIC model. But when SRIOV is used, VM connectivity has about the same performance has a real physical NIC, whereas with Virtio, performance could be poor.

Also, SR-IOV, by providing direct access to the physical NIC, is making host virtual network nodes (virtual router/switch) used by SDN solution totally blind about the VM using such connectivity. Local traffic switching between the VM connected on a same SR-IOV physical card is achieved by the virtual Ethernet bridge proposed by SR-IOV. Communication between VMs connected onto the distinct SRIOV physical ports must rely on physical network.

SDN vswitch/vrouter usage is very limited when SR-IOV is used. Indeed, packet switching between VMs which are using VFs of a same SR-IOV physical port are using the physical virtual Ethernet bridge hosted in the physical NIC. See Figure 2.9.

Only a few use cases are relevant, the first provides internal connectivity between VMs using distinct SR-IOV physical ports (it avoids sending the traffic out of the server to be processed by the physical network). See Figure 2.9.

Figure 2.9	xxxxxxxxxx. Zz zz. z

A second use case is building hybrid mode solutions with multi-NIC VM. Network traffic not requiring high performance uses emulated NIC (management traffic for instance). Network connectivity requiring high performance will be processed by the SR-IOV assigned NIC (for instance video data traffic).

Figure 2.10	Building Hybrid Mode Solutions with Nulti-NIC VM

With SR-IOV you get high performance but with poor flexibility and no network virtualization features. With VirtIO you get a high level of network virtualization suitable for SDN, which is very flexible with poor performances.

For SDN use cases, you need network virtualization features and performance. DPDK will bring both.

## Network Packer Processing Performance Requirements

The Ethernet minimum frame size is 64 Bytes. When Ethernet frames are sent onto the wire, Inter Frame Gap and Preamble bits are added. The minimum size of Ethernet frames on the physical layer is 84 Bytes (672 bits).

Figure 2.11	Packet Frame xxxxxxx

For a 10 Gbit/s interface, the number of frames per seconds can reach up to 14.88 Mpps for traffic using the smallest Ethernet frame size. It means a new frame will have to be forwarded every 67 ns.

A CPU running at 2Ghz has a 0.5 ns cycle. Such a CPU has a budget of only 134 cycles per packet to be able to process a flow of 10 Gb/s.

Generic Linux Ethernet drivers are not powerful enough to be able to process such a 10Gb/s packet flow. Indeed, with regular Linux NIC drivers lots of time is required to perform packet processing in Linux Kernel using interrupt mechanism, and transfer application data from host memory to a NIC.

DPDK is one of the best solutions available allowing you to build a network application using high-speed NICs and working at wire speed. Therefore, Contrail is proposing DPDK as one of the solutions to be used for the physical compute connectivity.

## DPDK and Network Applications

### DPDK application working principle

The Data Plane Development Kit (DPDK) is dedicating one (or more) CPUs to one (or more) threads that are continuously polling a one (or more) DPDK NIC RX queue. CPUs on which a DPDK polling thread is started will be loaded at 100% whether some packets are processed or not, as no interrupt mechanism is used in DPDK to warn the DPDK application that a packet has been received. (See Figure 2.12.)

Figure 2.12	DPDK Working Principle

Using the DPDK library API, physical NIC packets will be made available in user space memory in which the DPDK application is running. So, when DPDK is used, there is no user space to kernel space context switching and it saves lots of CPU cycles. Also, the host memory is using large continuous memory area, the huge pages, which allow large data transfers and avoid high data fragmentation in memory which would require a higher memory management effort at the application level. Such a fragmentation would also cost some precious CPU cycles.

Hence, most of the CPU cycles of the DPDK pinned CPU are used for polling and processing packets delivered by the physical NIC in DPDK queues. As a result, the packet forwarding task can be processed at a very high speed. If one CPU is not powerful enough to manage incoming packets that are hitting the physical NIC at a very high rate, you can allocate an additional one to the DPDK application in order to increase its packet processing capacity.

A DPDK application is a multi-thread program that is using the DPDK library to process network data. In order to scale, you can start several packet polling and processing threads (each one pinned on a dedicated CPU) that are running in parallel.

Three main components are involved in a DPDK application (see Figure 2.13):

- Physical NIC

  - buffering packets in physical queues

  - using DMA to transfer packets in host memory

- DPDK NIC abstraction with its queue representation in huge pages host memory:

  - descriptor rings

  - mbuf (to store packets)

- Linux pThread use to poll and process packets received in DPDK NIC queues.

Figure 2.13	Three Components of DPDK

### DPDK Overview

DPDK is a set of data plane libraries and network interface controller drivers for fast packet processing, currently managed as an open-source project under the Linux Foundation. The main goal of the DPDK is to provide a simple, complete framework for fast packet processing in data plane applications.

The framework creates a set of libraries for specific environments through the creation of an Environment Abstraction Layer (EAL), which may be specific to a mode of the Intel architecture (32-bit or 64-bit), Linux* user space compilers or a specific platform. These environments are created through the use of make files and configuration files. Once the EAL library is created, the user may link with the library to create their own applications.

The DPDK implements a “run to completion model” for packet processing, where all resources must be allocated prior to calling data plane applications, running as execution units on logical processing cores.

The model does not support a scheduler and all devices are accessed by polling. The primary reason for not using interrupts is the performance overhead imposed by interrupt processing.

For more information please refer to dpdk.org documents: http://dpdk.org/doc/guides/prog_guide/index.html

### DPDK Software Architecture

DPDK is a set of programing libraries that can be used to create an application that needs to process network packets at a high speed. DPDK has the following functions:

- A queue manager implements lockless queues

- A buffer manager pre-allocates fixed size buffers

- A memory manager allocates pools of objects in memory and uses a ring to store free objects

- Poll mode drivers (PMD) are designed to work without asynchronous notifications, reducing overhead

- A packet framework made up of a set of libraries that are helpers to develop packet processing

In order to reduce the Linux user to kernel space context switching, all these functions are made available by DPDK in to the user space where applications are running. User applications using DPDK libraries have a direct access to the NIC cards, without passing through a NIC Kernel driver as it is required when DPDK is not used.

| Regular Network Application image | Network Application with DPDK Image |
| --- | --- |

Figure 2.14	This Image is Missing??????

DPDK allows you to build user-space multi-thread network applications using the POSIX thread (pthread) library.

DPDK is a framework which is made of several libraries:

- Environment Abstraction Layer (EAL)

- Ethernet Devices Abstraction (ethdev)

- Queue Management (rte_ring)

- Memory Pool Management (rte_mempool)

- Buffer Management (rte_mbuf)

- Timer Manager (librte_timer)

- Ethernet Poll Mode Driver (PMD)

- Packet Forwarding Algorithm made up of Hash (librte_hash) and Longest Prefix Match (LPM,librte_lpm) libraries

- IP protocol functions (librte_net)

The ethdev library exposes APIs to use the networking functions of DPDK NIC devices. The bottom half part of ethdev is implemented by NIC PMD drivers. Thus some features may not be implemented.

Poll Mode Ethernet Drivers (PMDs) are a key component for DPDK. These PMDs bypass the kernel and provide direct access to the Network Interface Cards (NIC) used with DPDK.

Linux user space device enablers (UIO or VFIO) are provided by the Linux Kernel and are required to run DPDK. They are allowing PCI devices to discover and expose information and address space through the /sys directory tree.

DPDK libraries (See Figure 2.14) are allowing kernel-bypass application development:

- probing for PCI devices (attached via a Linux user space device enabler),

- huge-page memory allocation,

- data structures geared toward polled-mode message-passing applications:

  - such as lockless rings

  - memory buffer pools with per-core caches.

Figure 2.14	Overview of DPDK Libraries.

Only a few libraries have been described in this diagram: Set of libraries is enriched at each new DPDK release ( see https://www.dpdk.org/).

### DPDK Environment Abstraction Layer

The Environment Abstraction Layer (EAL) is responsible for providing access to low-level resources such as hardware and memory space. It provides a generic interface that hides the environment specifics from the applications and libraries. The EAL performs physical memory allocation using mmap() in hugetlbfs (using the huge page sizes to increase performance).

Provided services by EAL are:

- DPDK loading and launching

- Support for multi-process and multi-thread execution types

- Core affinity/assignment procedures

- System memory allocation/de-allocation

- Atomic/lock operations

- Time reference

- PCI bus access

- Trace and debug functions

- CPU feature identification

- Interrupt handling

- Alarm operations

- Memory management (malloc)

Figure 2.15	Environment Abstraction Layer (EAL).

## DPDK Memory Management

### DPDK optimized memory management for speed

DPDK has a highly optimized memory manager that works on a group of fixed size objects called a mempool. Every one of them are pre-allocated. DPDK does not encourage dynamic allocations because it consumes a lot of CPU cycles and it is a speed killer.

DPDK stores incoming packets into mbufs (memory buffers). DPDK pre-allocates a set of mbufs and keeps it in a pool called mempool. DPDK makes use of mempools each time it needs to allocate a mbuf where packets are stored. Instead of allocating a single mbuf, DPDK does a bulk allocation, or bulk free once packets are consumed. By doing this, packets to be processed (mbufs) are already in cache memory. Therefore, DPDK is very cache friendly.

Mempool has further optimizations. It is also very cache friendly. Everything is aligned to the cache and has a some mbufs allocated for each DPDK thread or lcore. Each mempool is bound with rings which are referencing mbufs containing packets stored in mempool.

Each ring is a highly optimized lockless ring. It can be used by several lcores in a multi-producer/multi-consumer kind of scenario without locks. By avoiding locks, DPDK gets large performance gains, as data structures locking is also a speed killer.

### mbufs and mempools

Network data is stored in compute central memory (in huge page area). DPDK uses message buffers known as mbufs to store packet data into the host memory. These mbufs are stored in memory pools known as mempools.

### Figure 2.16	mbufs and mempools

The mbufs are storing DPDK NIC incoming and outgoing packets which have to be processed by the DPDK application.

### Packet descriptors

DPDK queues are not storing the packets but a descriptor points to the real packet. It avoids performing a data transfer that would be needed when packets have to be forwarded from a DPDK NIC to another.

### Figure 2.17	Packet Descriptors

Packets are not moved from one queue to another, these are the descriptors (pointers) that are moving from one queue to another.

### Figure 2.18	Packet Descriptors MOVING

### DPDK rings

Descriptors are set up as a ring, or a circular array of descriptors. Each ring describes a single direction DPDK NIC queue. Each DPDK NIC queue is made up of two rings (one per direction: one RX ring, one TX ring).

### Figure 2.19	DPDK Rings

Each descriptor points onto a packet that has been received (RX ring) or that is going to be transmitted (TX ring). The more descriptors that RX/TX rings are containing, the more memory will be required in each mempool (number of mbufs) to store data.

### Data Transfer between host NIC and memory

The DPDK application is only processing packets that are exposed in user space host OS memory. DPDK rings are an abstraction of the real NIC queues: DPDK is using DMA to keep synchronized at any time between the NIC hardware queues and its DPDK representation in the host memory.

#### Physical NIC incoming packets

When an incoming packet is reaching the physical NIC interface, it is stored in NIC physical queue memory. The RX ring is managing packets that have to be processed by a DPDK application. Synchronization between the host OS and the NIC happens through two registers, whose content is interpreted as an index in the RX ring:

- Receive Descriptor Head (RDH): indicates the first descriptor prepared by the OS that can be used by the NIC to store the next incoming packet.

- Receive Descriptor Tail (RDT): indicates the position to stop reception, i.e. the first descriptor that is not ready to be used by the NIC.

### Figure 2.20	RDT Receive Descriptor Tail

DMA transfer is transparently copying packets from physical NIC memory to the host central memory. DMA is using the RDT descriptor as a destination memory address for the data to be transferred. Once packets have been transferred into host memory both RX rings and RDT are updated.

#### Physical NIC outgoing packets

When a packet has to be sent from host memory to the physical NIC interface, it is referenced in NIC TX ring by the DPDK application. TX ring is managing packets that have to be transferred onto a NIC card.

### Figure 2.21	TDT: Transmit Descriptor Tail

Synchronization between the host OS and the NIC happens through two registers, whose content is interpreted as an index in the TX ring:

- Transmit Descriptor Head (TDH): indicates the first descriptor that has been prepared by the OS and has to be transmitted on the wire.

- Transmit Descriptor Tail (TDT): indicates the position to stop transmission, i.e. the first descriptor that is not ready to be transmitted, and that will be the next to be prepared.

## DPDK and Packet Processing

### Linux pthreads

Multithreading is the ability of a CPU (single core in a multi-core processor architecture) to provide multiple threads of execution concurrently. In a multithreaded application, the threads share some CPU resources memory:

- CPU caches

- translation lookaside buffer (TLB)

A single Linux process can contain multiple threads, all of which are executing the same program. These threads share the same global memory (data and heap segments), but each thread has its own stack (local variables).

Linux pThreads (POSIX threads) is a C library which contains set functions that are allowing to manage threads into an application. DPDK is using the Linux pThreads library.

### DPDK lcores

DPDK is using threads that are designed as "lcore”. A lcore refers to an EAL thread, which is really a Linux pthread, which is running onto a single processor execution unit.

- first lcore: that executes the main() function and that launches other lcores is named master lcore.

- any lcore: that is not the master lcore is a slave lcore.

Lcores are not sharing CPU units. Nevertheless, if the host processor supports hyperthreading, a core may include several lcores or threads. Lcores are used to run DPDK application packet processing threads. Several packet processing models are proposed by DPDK. The simplest one is the Run-To-Completion model.

### Figure 2.22	Run-To-Completion

Run to completion is using a single thread (lcore) for end to end packet processing (packet polling, processing and forwarding).

### Multicore Scaling - Pipeline model

A complex application is typically split across multiple cores, with cores communicating through software queues. Packet framework facilitates the creation of pipelines. Each pipeline thread is assigned to a CPU and is using software queues like output or/and input ports.

### Figure 2.23	Run-To-Completion

For instance, Contrail DPDK vRouter is using such a model for GRE encapsulated packet processing.

### Control Threads

It is possible to create control threads. Those threads can be used for management/infrastructure tasks and are used internally by DPDK for multi process support and interrupt handling.

### Service Core

DPDK service cores enables a dynamic way of performing work on DPDK lcores. Service core support is built into the EAL and an API is provided to optionally allow applications to control how the service cores are used at runtime.

## DPDK and Poll Mode Drivers (PMD)

When DPDK is used, network interfaces are no longer managed in kernel space. The regular Linux NIC driver which is usually used to manage the NIC has to be replaced by a new driver which is able to run into user space. This new driver, called Poll Mode Driver (PMD), will be used to manage the network interface into user space with the DPDK library.

### Physical NIC and BAR registers

PCI devices have a set of registers referred to as configuration space for devices. These configuration space registers are mapped to host memory locations.

When a PCI device is enabled, the system’s device drivers (by writing configuration commands to the PCI controller) programs the Base Address Registers (BAR) to inform the PCI device of its address mapping. Next, the host operating system is able to address this PCI device.

### Linux NIC drivers

With usual Linux NIC kernels, both NIC configuration and packet processing is done in kernel space. User applications, which have to establish a TCP connection or send a UDP packet, use the sockets API exposed by libc library.

| image NIC configuration | image NIC packet processing |
| --- | --- |
| ### Figure 2.24 missing artwork? |  |

Linux packet processing with sockets API requires the following operations which can be costly:

- Kernel Linux system calls

- Multitask context switching on blocking I/O

- Data copying from kernel (ring buffers) to user space

- Interrupt handling in kernel

With usual Linux drivers most of operations are occurring in kernel modes and are requiring lots of user space to kernel space context switching and interruption mechanisms. The heavy context switching usage is costing lots of CPU cycles and is a limiting the numbers of packets that a CPU is able to process. Such drivers are not able to perform packet processing at expected high speeds, especially when 10/40/100G Ethernet generation cards are used on a Linux System.

### Poll Mode Drivers

A poll mode driver (PMD) consists of APIs running in user space to configure the devices and their respective queues. In addition, a PMD accesses the RX and TX descriptors directly without any interrupts (with the exception of Link status change interrupts) to quickly receive, process and deliver packets in the user’s application.

PMDs are involved in NIC configuration. They expose NIC configuration registers into host memory area which is directly reachable from user space.

| image NIC configuration | Image NIC packet processing |
| --- | --- |
| Figure 2.25 missing artwork? |  |

In shortPMDs are user space pthreads which:

- call specific EAL functions

- have a per NIC implementation

- have direct access to RX/TX descriptors

- use Linux user space device enablers (UIO or VFIO) driver for specific control changes (interrupts configuration)

Hence user applications can directly configure the NIC cards they are using from Linux user space where they are running.

A first configuration phase is using PMDs and the DPDK library to configure DPDK rings buffers into Linux user space. Next, incoming packets will be automatically transferred with DMA (Direct Memory Access) mechanism from the NIC physical RX queues in NIC memory to DPDK RX rings buffered in host memory. DMA is also used to transfer outgoing packets from the DPDK TX rings buffer in host memory to the NIC physical TX queues in NIC memory. DMA offloads expensive memory operations, such as large copies or scatter-gather operations, from the CPU.

### Direct Memory Access

DMA allows PCI devices to read (write) data from (to) memory without CPU intervention. This is a fundamental requirement for high-performance devices.

DMA is a mechanism that is using a specific hardware controller to manage read and write operations into the main system memory (RAM). This mechanism is totally independent of the CPU and does not consume any CPU resource. A DMA transfer is used to manage data transfer. DMA transfer is triggered by the CPU and is working in the background using the specific hardware resource (a DMA controller).

DPDK rings and NIC buffers are synchronized with DMA. Thanks to this synchronization mechanism, DPDK applications can access transparently to NIC packets in user space reading or writing data in DPDK rings.

### IOMMU

Input–Output Memory Management Unit (IOMMU) is a memory management unit (MMU) that connects a DMA capable I/O bus to the main memory.

In virtualization, an IOMMU is re-mapping the addresses accessed by the hardware into a similar translation table that is used to map guest virtual machine address memory to host-physical addresses memory.

Figure 2.26	IOMMU

IOMMU provides a short path for devices to get access only to a well-scoped physical device memory area which corresponds to a given guest virtual machine memory. IOMMU helps to prevent DMA attacks that could be originated by malicious devices. IOMMU provides DMA and interrupt remapping facilities to ensure I/O devices behave within the boundaries they’ve been allotted.

Intel has published a specification for IOMMU technology: Virtualization Technology for Directed I/O abbreviated as VT-d.

In order to get IOMMU enabled:

- both kernel and BIOS must support and be configured to use IO virtualization (such as VT-d).

- IOMMU must be enabled into Linux Kernel parameters in /etc/default/grub and run update-grub command.

GRUB configuration example with IOMMU Passthrough enabled:

GRUB_CMDLINE_LINUX_DEFAULT="iommu=pt intel_iommu=on"

### DPDK Supported NICs

The DPDK library includes PMDs for physical and emulated Ethernet controllers which are designed to work without asynchronous, interrupt-based signaling mechanisms.

- Available DPDK PMD for physical NIC:

  - I40e PMD for Intel X710/XL710/X722 10/40 Gbps family of adapters http://dpdk.org/doc/guides/nics/i40e.html

  - IXGBE PMD http://dpdk.org/doc/guides/nics/ixgbe.html

  - Linux bonding PMD http://dpdk.org/doc/guides/prog_guide/link_bonding_poll_mode_drv_lib.html

- Available DPDK PMD for Emulated NIC:

  - DPDK EM poll mode driver supports emulated Intel 82540EM Gigabit Ethernet Controller (qemu e1000 device):
http://doc.dpdk.org/guides/nics/e1000em.html

  - Virtio Poll Mode driver for emulated VirtIO NIC
http://dpdk.org/doc/guides/nics/virtio.html

  - VMXNET3 NIC when VMWare hypervisors are used:
http://doc.dpdk.org/guides/nics/vmxnet3.html

Lots of other NICs are supported by DPDK:
 (cf: http://doc.dpdk.org/guides/nics/overview.html

Different PMDs may require different kernel drivers in order to work properly (cf Linux user space device enablers). Depending on the PMD being used, a corresponding kernel driver should be loaded and bound to the network ports.

It is also preferable that each NIC has been flashed with the latest version of NVM/firmware.

## Linux User Space Device Enablers

Most of PMD are using generic user space device enablers to expose physical NIC registers in user space into the host memory. Two space device enablers are widely used by the DPDK PMD and they are UIO and VFIO.

### UIO - User Space IO

Linux kernel version 2.6, introduced the User Space IO (UIO) loadable module. UIO is a kernel-bypass mechanism which provides an API that enables user space handling of legacy interrupts (INTx).

UIO has some limitations:

- UIO does not manage message-signaled interrupts (MSI or MSI-X).

- UIO also does not support DMA isolation through IOMMU.

UIO only supports legacy interrupts so it is not usable with SR-IOV and virtual hosts which require MSI/MSI-X interrupts.

Despite these limitations, UIO is well suited for use in VMs, where direct IOMMU access is not available. In such a situation, a guest instance user space process is not isolated from other processes in the same instance. But the hypervisor can isolate any guest instance from others or hypervisor host processes using IOMMU.

Currently, two UIO modules are supported by DPDK:

- Linux Generic (uio_pci_generic), which is the standard proposed UIO module included in the Linux kernel.

- DPDK specific (igb_uio) which must be compiled with the same kernel as the one running on the target.

DPDK specific UIO Kernel module is loaded with insmod command after UIO module has been loaded:

$ sudo modprobe uio
$ sudo insmod kmod/igb_uio.ko

While a single command is needed to load Linux generic UIO module:

$ sudo modprobe uio_pci_generic

DPDK specific UIO module could be preferred in some situation to Linux Generic UIO module (cf: https://doc.dpdk.org/guides/linux_gsg/linux_drivers.html)

### VFIO – Virtual Function I/O

Virtual Function I/O (VFIO) kernel infrastructure was introduced in Linux version 3.6.

VFIO provides a user space driver development framework allowing user space applications to interact directly with hardware devices by mapping the I/O space directly to the application’s memory. VFIO is a framework for building user space drivers that provides:

- Mapping of device’s configuration and I/O memory regions to user memory

- DMA and interrupt remapping and isolation based on IOMMU groups

- Eventfd and irqfd based signaling mechanism to support events and interrupts from and to the user space application.

VFIO exposes APIs that allow:

- create character devices (in /dev/vfio/)

- support ioctl calls

- support mechanisms for describing and registering interrupt notification

VFIO driver is an IOMMU/device agnostic framework for exposing direct device access to user space, in a secure, IOMMU protected environment. For bare-metal environments, VFIO is the preferred framework for Linux kernel-bypass. It operates with the Linux kernel’s IO.

Figure 2.27	VFIO driver

MMU subsystem is used to place devices into IOMMU groups. User space processes can open these IOMMU groups and register memory with the IOMMU for DMA access using VFIO ioctl calls. VFIO also provides the ability to allocate and manage message-signaled interrupt vectors.

A single command is needed to load VFIO module:

$ sudo modprobe vfio_pci

Despite the fact that VFIO has been created to work with IOMMU, VFIO can be also be used without (this is just as unsafe as using UIO).

### Linux user space device enablers to be used

VFIO is generally the preferred Linux user space device enabler to be used because it supports IOMMU to protect host memory. When a real hardware PCI device is attached to the host system and IOMMU is used with VFIO, all the reads/writes of that device done in user space by the DPDK application will be protected by the host IOMMU.

But there are a few exceptions. Figure 2.28 is an Intel recommendation for the choice of the Kernel driver to be used with DPDK (https://software.intel.com/content/www/us/en/develop/articles/memory-in-dpdk-part-2-deep-dive-into-iova.html).

Figure 2.28	Choice of the Kernel driver to be used with DPDK

## DPDK and Host Hardware Architecture

### NUMA

NUMA means Non-Uniform Memory Access systems and consists of a traditional server that has a single CPU, a single RAM, and a single RAM controller.  RAM can be made of several DIMM banks in several sockets, all being associated to the CPU. When the CPU needs access to data in RAM, it requests to its RAM controller. For example, in a server with two CPUs, each one can be a separate NUMA: NUMA0 and NUMA1, as shown in Figure 2.30.

Figure 2.29	NUMA Overview

NUMA nodes architecture in Figure 2.29 shows:

- In green: CPU core accessing a memory item located in its own NUMA’s RAM controller, showing minimum latency.

- In red: CPU core accessing a memory item located in the other NUMA through the QPI (Quick Path Interconnect) path and the remote RAM controller, showing a higher latency.

When CPU0 needs to access data located in RAM0, it will go through its local RAM controller 0. Same thing happens for CPU1.

When CPU0 needs to access data located in the other RAM1, the first (local) controller 0 has to go through the second (or remote) RAM controller 1 which will access the (remote) data in RAM 1. Data will use an internal connection between the two CPUs called QPI, or Quick Path Interconnect, which is typically of a high enough capacity to avoid being a bottleneck, typically one or two times 25GBps (400 Gbps). For example, the Intel Xeon E5 has two CPUs with two QPI links between them; Intel Xeon E7 has four CPUs, with a single QPI between pairs of CPUs.

The fastest RAM that the CPU has access to is the register, which is inside the CPU and reserved for it. Beyond the register, the CPU has access to cached memory, which is a special memory based on higher performance hardware (see Figure 2.30).

Figure 2.30	Numa Nodes Overview

Cached memories are shared between the cores of a single CPU. Typical characteristics of memory cache are:

- Accessing a Level 1 cache takes seven CPU cycles (with a size of 64KB or 128KB).

- Accessing a Level 2 cache takes eleven CPU cycles (with a size of 1MB).

- Accessing a Level 3 cache takes thirty CPU cycles (with a larger size).

If the CPU needs to access data that is in the main RAM, it has to use its RAM controller.

Access to RAM takes typically 170 CPU cycles (the green line in Figure 2.29). Access to the remote RAM through the remote RAM controller typically adds 200 cycles (the red line in Figure 2.29), meaning RAM latency is roughly doubled.

When data needed by the CPU is located both in the local and in the remote RAM with no particular structure, latency to access data can be unpredictable and unstable.

### Hyper-threading (HT)

A single physical CPU core with hyper-threading appears as two logical CPUs to an operating system. While the operating system sees two CPUs for each core, the actual CPU hardware only has a single set of execution resources for each core. Hyper-threading allows the two logical CPU cores to share physical execution resources.

The sharing of resources allows two logical processors to work with each other more efficiently and allows a logical processor to borrow resources from a stalled logical core (assuming both logical cores are associated with the same physical core). Hyper-threading can help speed processing up, but it’s nowhere near as good as having actual additional cores.

### Huge Pages

Memory is managed in blocks known as pages. On most systems, a page is 4KB, and 1MB of memory is equal to 256 pages; 1GB of memory is 256,000 pages, etc. CPUs have a built-in memory management unit that manages a list of these pages in hardware.

Figuree 2.31	Huge Pages Memory Allocation

The Translation Lookaside Buffer (TLB) is a small hardware cache of virtual-to-physical page mappings. If the virtual address passed in a hardware instruction can be found in the TLB, the mapping can be determined quickly. If not, a TLB miss occurs, and the system falls back to slower, software-based address translation. This results in performance issues. Since the size of the TLB is fixed, the only way to reduce the chance of a TLB miss is to increase the page size.

Figure 2.32	Huger Pages Memory Management

Virtual memory address lookup slows down when the number of entries increases.

A huge page is a memory page that is larger than 4Ki. In x86_64 architecture, in addition to standard 4KB memory page size, two larger page sizes are available: 2MB and 1GB. Contrail DPDK vrouter can use both or only one huge page size.

### CPU Isolation and Pining

An operating system is using a scheduler to place each single process and/or threads it has to run onto one of the CPUs offered by a host. There are two kinds of scheduling, cooperative and preemptive. By default, Linux scheduler is using a cooperative mode.

In order to get a CPU booked for a subset of tasks, you have to inform the OS scheduler not to use these CPUs for all the tasks it has to run. These CPUs are "isolated" because they are no longer used by the OS to process all the tasks. In order to get a CPU isolated several mechanisms can be used:

- remove this CPU from the common CPU list used to process all tasks

- change the scheduling algorithm (cooperative to preemptive)

- participate or do not interrupt processing

Isolation and pinning are two complementary mechanisms that are proposed by Linux OS:

- CPU isolation restricts the set of CPUs that are available for operating system scheduler level. When a CPU is isolated, no task will be scheduled on it by the OS. An explicit task assignment must be done.

- CPU pinning is also called processor affinity and it enables the binding and unbinding of a process or a thread onto the CPU. On the opposite side, CPU pinning is a mechanism that consists in defining a limited set of CPUs that are allowed to be used by:

  - the OS scheduler. OS CPU affinity is managed through systemd

  - a specific process: using CPU pinning rules (taskset command for instance)

Tasks to be run by an operating system must be spread across available CPUs. These tasks are in a multi-threading environment often made of several processes which are also made of several threads.

### CPU Isolation Mechanisms

#### isolcpus

Isolcpus is a Kernel scheduler option. When a CPUs is specified in isolcpus list, it is removed from the general kernel SMP balancing and scheduler algorithms. The only way to move a process onto or off OF an isolated CPU is via the CPU affinity syscalls (or to use the taskset command).

This isolation mechanism:

- removeS isolated CPUs from the common CPU list used to process all tasks

- change the scheduling algorithm from cooperative to preemptive

- performS CPU isolation at the system boot

When isolcpus is suffering of lots of drawbacks:

- it requires manual placement of processes on isolated CPUs.

- it is not possible to rearrange the CPU isolation rules after the system startup

- the only way to change isolated CPU list is by rebooting with a different isolcpus value in the boot loader configuration (GRUB for instance).

- isolcpus is disabling the scheduler load balancer for isolated CPUs. It also means the kernel will not balance those tasks equally among all the CPUs sharing the same isolated CPUs (having the same affinity mask)

#### CPU Shield

The cgroups subsystem is proposing a mechanism to dedicate some CPUs to one or several user processes. It consists in defining a user shield group which is protecting a subset of CPU system tasks.

Three cpusets are defined:

- root: present in all configurations and contains all CPUs (unshielded)

- system: contains CPUs used for system tasks - the ones which need to run but aren’t important (unshielded)

- user: contains CPUs used for tasks that you want to assign a set of CPUs for their exclusive use (shielded)

The CPU shield can be manipulated with cset shield command.

#### Tuned

Tuned is a system tuning service for Linux. Tuned is using Tuned profiles to describe Linux OS performance tuning configuration.

The cpu-partitioning profile partitions the system CPUs into isolated and housekeeping CPUs. This profile is intended to be used for latency-sensitive workloads.

NOTE	 Tuned is only supported on Linux RedHat OS family. See : https://tuned-project.org/.

### Linux systemd - System task CPU affinity

A thread’s CPU affinity mask determines the set of CPUs on which it is eligible to run. Linux systemd is a software suite that provides an array of system components for Linux operating systems. Its primary component is an init system used to bootstrap user space and manage user processes. The CPUAffinity parameter restricts all processes spawned by systemd to the list of cores defined by the affinity mask.

#### default CPU affinity

When run as a system instance, systemd interprets the configuration file /etc/systemd/system.conf. In this configuration file, CPUAffinity variable configures the CPU affinity for the service manager as well as the default CPU affinity for all forked off processes.

#### Per service specific CPU affinity

Individual services may override the CPU affinity for their processes with the CPUAffinity setting in unit files:

# vi /etc/systemd/system/<my service>.service
...
[Service]
CPUAffinity=<CPU mask>

If a specific CPUAffinity has been defined for a given service, it has to be restarted in order for the new configuration file to be taken into consideration.

### CPU assignment for user processes (taskset)

Taskset is used to set or retrieve the CPU affinity of a running process given its PID or to launch a new command with a given CPU affinity. You can retrieve the CPU affinity of an existing task by:

# taskset -p pid

Or set it:

# taskset -p mask pid

## Bind a Virtual NIC to DPDK

DPDK requires a direct NIC access into user space. VirtIO vhost-user backend is exposing the Virtio network device in user space. The vhost-user is a library that implements the vhost protocol in user space. The vhost-user library allows you to expose a Virtio backend interface into user space.

The vhost-user library defines the structure of messages that are sent over a UNIX socket to communicate with the Virtio net device backend (vhost-net kernel driver is using ioctls instead)(see Figure 2.33).

Figure 2.33	Vhost-user Library

You can see the kernel-mode VM connected to a DPDK compute application. The user application is using both:

- the vhost user library: for emulated PCI NIC control plane

- the DPDK libraries: for emulated PCI NIC data plane

Support for user space vhost has also been provided with QEMU 2.1.

## Run DPDK in a guest VM

### Virtual IOMMU

Virtual IOMMU (vIOMMU) allows emulation of the IOMMU for guest VMs.

vIOMMU has the following characteristics:

- Translates guest VM I/O Virtual Addresses (IOVA) to Guest Physical Addresses (GPA)

- Guests VM Physical Addresses (GPA) are translated to Host Virtual Addresses (HVA) through the hypervisor memory management system.

- Performs device isolation.

- Implements a I/O TLB (Translation Lookaside Buffer) API which exposes memory mappings

In order to get a virtual device working with a virtual IOMMU you have to create the needed IOVA mappings into the vIOMMU and configure the device’s DMA with the IOVA.

The following mechanisms can be used to create vIOMMU memory mappings:

- Linux Kernel’s DMA API for kernel drivers

- VFIO for user space drivers (see Figure 2.34)

Figure 2.34	VFIO for user space drivers

The integration between the virtual IOMMU and any user space network application like DPDK is usually done through the VFIO driver. This driver will perform device isolation and automatically add the memory (IOVA -to GPA) mappings to the virtual IOMMU.

The use of huge pages memory in DPDK contributes to optimize TLB lookups, since fewer memory pages can cover the same amount of memory. Consequently, the number of device TLB synchronization messages drops dramatically. Hence, the performance penalty from TLB lookups is lowered, see: https://www.redhat.com/en/blog/journey-vhost-users-realm, and https://wiki.qemu.org/Features/VT-d.

### Virtio Poll Mode Driver

Virtio-pmd driver, is a DPDK driver, built on the PMD abstraction, that implements the virtio protocol, see Figure 2.35.

Figure 2.35	VFIO for user space drivers

Vhost user protocol moves the virtio ring from kernel all the way to user space. The ring is shared between the guest and DPDK application. QEMU sets up this ring as a control plane using UNIX sockets.

If the both the host server guest VM are DPDK ,there are no VMExits in the host for guest packets processing. Guest virtual machines use the virtio-net PMD driver which performs packets polling. So. There is nothing running in kernel here, so there are no system calls. Since both system calls and VM Exits are avoided, the performance boosts is significant.

### Physical Network Device Assignment (VFIO) and PCI passthrough

When a DPDK application is running into a guest VM, a mechanism has to be used to expose one of the host physical NICs to this guest in order that it gets access to the physical network.

IOMMU protects host memory against malicious or bug writes which can corrupt host memory at any time. But, when a physical device is assigned to a guest VM without vIOMMU usage, the guest memory address space is totally exposed to the hardware PCI device, see Figure 2.36.

Figure 2.36	Physical Device Assigned to a Guest VM Without vIOMMU

A PCI device can be assigned to a guest in order to be used by a guest DPDK application. By leveraging the VFIO driver in the host kernel you can provide direct access to an assigned physical NIC from this guest protected by IOMMU.

Next, by leveraging the VFIO driver in the guest kernel you can provide a direct access to the assigned physical from this guest user space. Thre vIOMMU is providing a secure mechanism to manage DMA transfer between an assigned physical hardware and hosted guest virtual instance memory area.

### SRIOV and DPDK in Guest VM

This use case is almost the same as PCI passthrough but VFIO and IOMMU are used to expose a SRIOV virtual function directly to a guest VM.

An additional a physical function driver, which is vendor specific, is used to manage the virtual function creation on the physical NIC. This driver is used by a VMM (like libvirt) to create the virtual function before the virtual instance is spawned, see Figure 2.37.

Figure 2.37	creating the virtual function before the virtual instance is spawned

In Figure 2.37, physical incoming packets are directly copied in guest memory without involving the host server. SR-IOV only allows to share a physical NIC between several guests but does not change the packet processing path provided by PCI passthrough.

## Virtio-assisted Hardware Acceleration

With DPDK and Virtio we have a technology that is allowing to get network virtualization at a high speed. This is a key technology for the SDN data plane. But this packet processing model has still some drawbacks, mainly:

- DPDK requires isolating some host CPUs for its exclusive use, meaning less CPU resources for the user application.

- Compute CPUs are generic and are not optimized for packet processing. DPDK is requiring lots of CPU usage to provide both fa eature rich and performance virtual network (host compute for DPDK vrouter/vswich application and on guest VMs for DPDK end-user applications).

SR-IOV adds performance but it’s use is limited in SDN applications due to its direct path between gthe uest VM and the NIC hardware that bypasses the host operating system in which the SDN network function is running (vswitch and vrouter).

In the coming sections, we will describe some evolving features on both the Virtio and direct device assignment to provide a solution that:

- is running in user space, like proposed by DPDK

- with good hardware performance, like that proposed by SRIOV and direct physical device assignment

- can be used in SDN, like that nproposed by Virtio software solution.

### Virtio Full Offloading

With Virtio full hardware offloading, both the Virtio data plane and Virtio control plane are offloaded to the NIC hardware. The physical NIC must support:

- the Virtio control specification: discovery, feature negotiation, establishing/terminating the data plane.

- the Virtio data plane specification: Virtio ring layout.

Hence once the guest memory is mapped with the NIC using Virtio physical device passthrough, the guest communicates directly with the NIC via PCI without involving any specific drivers in the host kernel.

Guest VM packet processing is directly performed in NIC hardware but presented to the guest instance like a regular Virtio-emulated interface. Guest VM does not make any difference between a Virtio-emulated interface and an assigned physical Virtio NIC, as they are exposed with the same Virtio driver frontend in the guest. See Figure 2.38.

Figure 2.38	Virtio full hardware offloading

### Virtio Device Passthrough

Virtio device passthrough can be implemented on a NIC which is supporting or not supporting SR-IOV. Like other physical device assignment technology presented in this book, VFIO and IOMMU are used to present the physical device NIC into the guest VM user space.

Hence, such a Virtio physical NIC can be used by a DPDK application running in a virtual instance. But, like other Virtio device passthrough, this has also the same limitations for SDN. As the host operating system is totally by-passed by this mechanism, you cannot interconnect instances using such NIC interfaces with a SDN virtual router or switch.

The main advantage of Virtio device passthrough is the flexibility it provides for a virtual instance to transparently use either a real physical interface or an emulated one. It offers an open public specification, which supports devices fully independent of any specific vendor.

Virtio full HW offloading, can support live migration thanks to Virtio, which is not possible to achieve without any specific implementation with SR-IOV. But in order to be able to support such a feature, the latest Virtio specifications (1.1 version) must be implemented onto both the QEMU and the NIC hardware used on the cloud infrastructure.

Figure 2.39	Virtio device passthrough

### Virtio Datapath Acceleration

Like full hardware offloading, virtual Data Path Acceleration (vDPA) aims to standardize the physical data plane using the virtio ring layout and present a standard Virtio driver in the guest decoupled from any vendor implementation for the control path.

vDPA is presenting a generic control plane through software which provides an abstraction layer on top of physical NIC. Like Virtio full hardware offloading, vDPA build sa direct data path between the guest network interface and the physical NIC, using the Virtio ring layout. But for the control path a generic vDPA driver (mediation driver) is used to translate the vendor NIC driver/control plane to the Virtio control plane, in order to allow each NIC vendor to keep using its own driver.

vDPA allows NIC vendors to support Virtio ring layout with smaller effort keeping wire speed performance on the data plane. See Figure 2.40.

Figure 2.40	Virtual Data Path Acceleration

vDPA is requiring a vendor specific mediation device driver to be loaded in the host operating system as shown in Figure 2.41.

Figure 2.41	Mediation Device Driver being loaded in the host OS

### Smart NIC

A NIC card generator, commonly named smart NIC, is highly customizable thanks to the last evolution provided by some new capabilities (FPGA, ARM, P4). It’s now possible to envisage SDN vSwitch/vRouter data plane functions to be moved onto the NIC card keeping only the control plane function in the host operating system.

For Contrail solutions, this is made by offloading several Contrail vRouter tables including:

- Interface Tables

- Next Hop Tables

- Ingress Label Manager (ILM) Tables

- IPv4 FIB

- IPv6 FIB

- L2 Forwarding Tables

- Flow Tables

It allows to accelerate lookups and forwarding actions that are directly performed into the NIC, as shown in Figure 2.42.

Figure 2.42	 Smart NIC

You can see in Figure 2.42 that SDN packet processing is fully done on the NIC card and no more host CPU processing is involved in packet processing.

Two implementations are proposed by Netronome are SRIOV + SmartNIC, and XVIO + Smart NIC, as shown in Figures 2.43 and 2.44.

Figure 2.43	 SRIOV + SmartNIC

Figuree 2.44	XVIO + Smart NIC

### eBPF and XDP

Berkeley Packet Filter (BPF) was designed for capturing and filtering network packets that matched specific rules. In the past few years extended BPF (eBPF) has been designed to take advantage of new hardware (64 bits usage for instance). An eBPF program is attached to a designated code path in the kernel.

eXpress Data Path (XDP), uses eBPF to achieve high-performance packet processing by running eBPF programs at the lowest level of the network stack, immediately after a packet is received by XDP. See Figure 2.45.

Figure 2.45	eBPF and XDP

XDP support is made available in the Linux kernel since version 4.8, while eBPF is supported in the Linux kernel since version 3.18.

XDP requires:

- MultiQ NICs

- Common protocol-generic offloads:

  - TX/RX checksum offload

  - Received Side Scaling

  - Transport Segmentation offload (TSO)

XDP packet processor performs:

- In Kernel RX packets processing

- Process RX packets directly (without any additional memory allocation for software queue, nor socket buffer allocation)

- Assign one CPU to each RX queue. This CPU can be configured into poll mode or interrupt mode.

- Trigger BPF program for packet processing

BFP programs:

- parse packets

- perform table lookup

- manage stateful filters

- manipulate packets (encapsulation, decapsulation, NAT, …)

BFP program main actions are:

- Forward

- Forward after modification (NAT)

- Drop

- Normal receive (regular Linux packet processing with socket buffer and TCP/IP stack)

- Generic Receive Offload (coalesce several received packets of a same connection

XDP is also able to offload an eBPF program to a NIC card which supports it, reducing the CPU load. See Figure 2.46.

Figure 2.46	XDP Offloads eBPF programs to a NIC card

XDP and eBPF do not require:

- allocation of large pages

- allocation of dedicated CPUs

- choosing packet polling or interrupt driven networking model

- user space to kernel space context switching to perform eBPF filtering

- allowing packet processing offload when supported by used NIC card

NOTE	 eBPF rules are also supported in DPDK application. See: https://www.redhat.com/en/blog/using-express-data-path-xdp-red-hat-enterprise-linux-8

## NIC Virtualization Solutions Summary

You’ve seen lots of NIC virtualization models for virtual instances. From a full software implementation like that proposed by Virtio to fully hardware assisted solutions like that proposed by SR-IOV. Also, DPDK is providing the ability to move NIC packet processing from Kernel space to user space.

Figure 2.47 provides an overview of the NIC virtualization solution:

- Full software solutions are very flexible and fit well with SDN and cloud feature expectation (such as live migration, east-west traffic inside host computes)

- Hardware assisted solutions perform well but fit less with expected virtualization flexibility. Guest VM migration is poorly supported due to hardware dependencies. These solutions fit well with applications requiring a huge north-south traffic (from guest WM to cloud outside).

Figure 2.47	Overview of the NIC Virtualization Solution

In the middle of this matrix chart, SmartNIC and DPDK are offering the best compromise for SDN usage. Smart NICs propose  very high performance, but this is still not a fully mature solution (lots of implementations vendor specific, but no agreed standards). Figure 2.48 lists the feature sets of this chapter’s examinations.

(*): depends on hardware and QEMU latest virtio specification support on the NIC card.

Figure 2.48: Table of Feature Sets