# vRouter DPDK (Prototype)

# Performance of MPLSoGRE traffic using Intel Dynamic Device Personalization (DDP) Technology for Fortville (X710) NICs

Authors: Kiran KN <kirankn@juniper.net>,

Przemysław Grygiel <pgrygiel@juniper.net>

# Specifications

## Hardware

| Model name | Intel(R) Xeon(R) Gold 5120 CPU @ 2.20GHz |
| --- | --- |
| CPUs | 56 |
| Sockets | 2 |
| Cores per Socket | 14 |
| Threads per core | 2 |
| NUMA 0 CPU list | 0,2,4,6,8,10,12,14,16,18,20,22,24,26, 28,30,32,34,36,38,40,42,44,46,48,50,52,54 |
| NUMA 1 CPU list | 1,3,5,7,9,11,13,15,17,19,21,23,25,27, 29,31,33,35,37,39,41,43,45,47,49,51,53,55 |

## Configuration of vRouter on compute-B (DUT):

| Operating System | RedHat 7.7 |
| --- | --- |
| Number of Forwarding Cores | 6 (Physical) + 6 (Siblings) |
| Drop Rate | 0.001% |
| Bond | 2 x 10G (802.3ad LACP) with L3/L4 hash (Intel x710) |
| Hugepages | 1G |
| NUMA allocation | vRouter, NIC and VM on the same socket (NUMA 0) |
| Contrail version | 2003 GA |

## CPU allocation (Full-CPU partitioning scheme) on compute-B:

| vRouter forwarding cores | 2,4,6,8,16,18,30,32,34,36,44,46 (6pcores/12 lcores) |
| --- | --- |
| Service cores | 0,1,28,29 |
| Control cores | 0,1,28,29 |
| Host OS cores | 0,1,28,29 |
| Kernel isolcpus and tuned isolcpu list | 2-27,30-55 |

## Traffic pattern

| Number of flows | 16535 |
| --- | --- |
| Encapsulation | MPLSoGRE, MPLSoUDP |
| Packet size | 64B |
| Mode | Packet |

# Results

| Use case | Total Mpps | Per core Mpps | Avg. Latency | Max Latency [us] |
| --- | --- | --- | --- | --- |
| MPLSoGRE without DDP | 6.486 | 1.081 | 311 | 4154 |
| MPLSoGRE with DDP | 11.230 | 1.871 | 181 | 768 |
| MPLSoUDP* | 11.140 | 1.857 | 162 | 720 |

## *DDP not needed as packets are hashed by NIC already

Adding hashing of MPLSoGRE on Intel X710 improves performance by a phenomenal 73% and latency by 58%.

## Details for non DDP for 6 + 6HT

+-------------------------------------------------------------------------------------------------------------------------------------------------------------------------+ ¦ UDP, 64 bytes, different number of flows by randomizing SRC & DST UDP port ¦ +--------+--------------------+----------------+----------------+----------------+------------------------+----------------+----------------+----------------+------------+ ¦ Flows ¦ Speed requested ¦ core generated ¦ Sent by Gen NIC¦ Forward by SUT ¦ core received ¦ Avg. Latency ¦ Max. Latency ¦ Packets Lost ¦ Loss Ratio ¦ +--------+--------------------+----------------+----------------+----------------+------------------------+----------------+----------------+----------------+------------+ ¦ 16384 ¦ 21.8% 3.244 Mpps ¦ 3.243 Mpps ¦ 3.243 Mpps ¦ 3.243 Mpps ¦ 2.2Gb/s 3.243 Mpps ¦ 311 us ¦ 4154 us ¦ 92 ¦ 0.00% ¦ +--------+--------------------+----------------+----------------+----------------+------------------------+----------------+----------------+----------------+------------+

## Results for the case with DDP for 6 + 6HT

+-------+-------------------------------------------------------------------------------------------------------------------------------------------------------------------------+ ¦ UDP, 64 bytes, different number of flows by randomizing SRC & DST UDP port ¦ +--------+--------------------+----------------+----------------+----------------+------------------------+----------------+----------------+----------------+------------+ ¦ Flows ¦ Speed requested ¦ core generated ¦ Sent by Gen NIC¦ Forward by SUT ¦ core received ¦ Avg. Latency ¦ Max. Latency ¦ Packets Lost ¦ Loss Ratio ¦ +--------+--------------------+----------------+----------------+----------------+------------------------+----------------+----------------+----------------+------------+ ¦ 16384 ¦ 37.7% 5.615 Mpps ¦ 5.615 Mpps ¦ 5.615 Mpps ¦ 5.615 Mpps ¦ 3.8Gb/s 5.615 Mpps ¦ 180 us ¦ 767 us ¦ 0 ¦ 0.00% ¦ +--------+--------------------+----------------+----------------+----------------+------------------------+----------------+----------------+----------------+------------+

## Results for the case with MPLSoUDP

+------- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------+ ¦ UDP, 64 bytes, different number of flows by randomizing SRC & DST UDP port ¦ +--------+--------------------+----------------+----------------+----------------+------------------------+----------------+----------------+----------------+------------+ ¦ Flows ¦ Speed requested ¦ core generated ¦ Sent by Gen NIC¦ Forward by SUT ¦ core received ¦ Avg. Latency ¦ Max. Latency ¦ Packets Lost ¦ Loss Ratio ¦ +--------+--------------------+----------------+----------------+----------------+------------------------+----------------+----------------+----------------+------------+ ¦ 16384 ¦ 37.4% 5.572 Mpps ¦ 5.572 Mpps ¦ 5.572 Mpps ¦ 5.572 Mpps ¦ 3.7Gb/s 5.572 Mpps ¦ 162 us ¦ 720 us ¦ 0 ¦ 0.00% ¦ ¦ ¦ Latency accuracy issue?: 12% ¦ +--------+--------------------+----------------+----------------+----------------+------------------------+----------------+----------------+----------------+------------+