Q: ping? "ring" = "descriptor", seems like located on both cores or NIC? also see vrouter DPDK fine tuning parameters.

A (LD): Ring is a set of memory pointers (descriptors). These pointers are pointing the real memory area in which the data to be processed (packets) are stored.

There are 2 rings per “queues” (1 per direction:. 1 RX ring, 1 TX ring).

Data are stored in compute central memory (in huge page area). Packets are not moved from one queue to another, but these are descriptors (pointers) that are moving from one queue to another.

Q: ping? vhost0 ?

A (LD): here is a drawing which is describing how are plugged each internal nic onto vrouter dataplane. Vhost0 is belonging to vrouter agent, not to vrouter dataplane. This is vif0/1 which is connected onto the vrouter dapaplane.

Q: ping? very confusing. one Q per cpu? is Q in NIC or in CPU?

DPDK application principle is simple …

We are allocating as many as needed CPUs as required to process incoming packets.

You have to consider the application we are building with DPDK.

For Juniper/Contrail this is a vrouter processing packets coming from a single physical Interface (to be sent to VM).

PS: This single interface can be a bond. But at vrouter level there is always a unique physical interface.

All the dimensioning consist in “how many CPU do I need to process packet coming from this interface (vif0) ?”

The answer is N CPU. For each CPU we are allocating 1 processing bidirectional Q (2 in reality because there  are 2 directions = 1 for RX and one for RX) which are the buffers in which we store the packets coming from the physical NIC (RX Q) or to sent to the physical NIC (TX Q).

So N CPU, means N bidirectional Q (in fact this is 2 x N single direction Q   N x RX + N  x TX).

Q ping: Figure 9. vrouter 4 topologies. ping? why vhost0?

It was a mistake. This is vif0.

Q: ping? how to print it in contrail5?

A (LD): Since contrail 5, we are using contrainers.

There is no more this single file used for vrouter config:

cat /etc/contrail/supervisord_vrouter_files/contrail-vrouter-dpdk.ini

Now, the parameters are in vhost0 interface configuration:

vi /etc/sysconfig/network-scripts/ifcfg-vhost0

CPU_LIST=0x154

DPDK_COMMAND_ADDITIONAL_ARGS=”--vr_nexthops=32768”

Q: ping? how is this neat output captured?

A (LD): This is a script I made in order to display (at least to have a representation – I’ve enriched a Premecz Python program) about packet processing. I did it on Premecz lab during performance tests we’ve made early this year.

https://drive.google.com/open?id=1mhxzRlulBqo3SfbGOywy2smG-NdC5dVN

Q: ping? no 1x1 mapping between vNIC queue and forwarding threads? as with pNIC?

A (LD): Yes, there is. But you have 2 cases to consider. SingleQ and MultiQ NIC.

Also, there is a balancing mechanism in order to attach queue0 of each NIC not on a same CPU (

Cf: vNIC queues - Polling Core Assignment (R1910)

Here, multiQ case:

Each CPU is bound to a Q :

But it is only part of the full view. You have to consider each direction … and type of traffic.

See my drawings.

Q: ping? confusing: #2 and #3 statements are conflicting

A (LD):

- The packet is placed on a vNIC TX queue (vRouter vif RX queue) by the Virtual machine.

- A given vNIC TX queue is always polled by the same vRouter logical core.

- A round robin algorithm (described in next section) is used to assign vNIC queues to logical cores.

- vNIC sub-interfaces are sharing TX/RX queues with their parent interface. Hence the same logical co

Not very well described indeed.

Step 3 is done once … at vnic plugin on vrouter

Step 2 is done during vnic life (how each packet is processed)

Q ping: per vrouter packet processing (first overview) what is meant here is there are 2 hash:

A (LD): First hash is calculated on underlay packet outer header by the NIC card.

Second hash is calculated by vrouter application on inner header (on decapsulated packet) because if the first hash result would be reused, we would have same selected CPU for packet processing than those used for packet polling.

This is this second hash calculation that is allowing to use a different CPU for packet processing than for packet polling and to spread the DPDK vrouter load on all CPUs when MPLSoGRE is used:

This is explained in this drawing. Packets are all polled by the first CPU (due to poor entropy).

Then due to second hash calculation, packets are re-balanced on the 3 others.

Q: ping? this is different than ingress traffic received from pNIC - only MPLSoGRE traffic will use a different forwarding thread than polling thread

A (LD): Yes. Indeed.

Yes, and this is something that will change in next releases. To be discussed with Kiran and Premecz. It seems we will provide the ability to disable this hash calculation done at vrouter level to rebalance polled packets onto different CPU to be processed.

Q: ping: this is the diff with MPLSoGRE, where we see "an extra hop"

A (LD): Yes. Indeed.

We have to simplify explanations … this is currently very confusing.

In fact this is not so complicated. You have two situations:

- first: processing core is not the same as polling core (there are 2 hash used – first calculated by pNIC or vNIC, second one by vrouter)

- second: processing and polling core are the same (we are using only 1 hash calculated by pNIC or vNIC)

Next we could provide a matrix stating when 1 or 2 hash are used, and illustrate some situation with diagrams.

Q ping? why diff between vNIC and pNIC? when pNIC got MPLSoUDP (just a UDP packet anyway), it do RSS hashing and put in differnet pNIC queue → diff polling core, so polling core does not need to hash again and just processed it. why vNIC do it differently? vNIC got UDP packet from VM, it should again do hash and place it into different queue, so different polling core will just poll and process it. why distribute to other cores here?

A (LD): First, I think this provided matrix has to be discussed and revised … Personally I do not understand this concept of “IPv4 MPLSoGRE” when packet is originated onto a vNIC.

My understanding is this situation :

Incoming packets from Physical NIC

| Outer packet Type | Inner packet type | Action |
| --- | --- | --- |
| IPv4 MPLSoUDP or VXLAN | any | Polling = Processing core (single hash) |
| IPv4 MPLSoGRE | IP (v4 or v6) | Polling != Processing core (two hash calc.) |
|  | Not IP (Ethernet Frame for instance) | Polling = Processing core (single hash) |
| IPv4 | Not encapsulated (like XMPP messages) | ?? |

Incoming packets from Virtual NIC

| Packet type | Action |
| --- | --- |
| IP (v4 or v6) | Polling != Processing core (two hash calc.) |
| Not IP (Ethernet Frame for instance) | Polling = Processing core (single hash) |

Second: Why all this complex stuff ?

This is (it was) required by poor entropy situations:

- MPLS GRE encapsulation (incoming traffic from other baremetal computes)

- SingleQ VM (incoming traffic from virtual instances)

In both situations only 1 polling core is selected. So it is useless to allocate lots of CPU onto your vrouter if due to poor entropy / or single Q VM, you are using only one core.

This is why Contrail has created a mechanism to spread the load on all CPU. This mechanism consist in different hash calculation in order to select another CPU for processing.

This was a nice solution till, GRE was the main encapsulation protocol supported and most of DPDK VM were single Q.

But now, as most of our customers are using UDP encapsulation protocols and MultiQ VNF, this old nice mechanism is really a drawback. It brings an additional calculation and an additional queuing steps that is badly impacting vrouter performances.

This is why in next release the idea is to propose a setup in which all packets are polled and processed by the same CPU. It will be worth to use this new setup when your VNF are MultiQ and when using UDP encapsulations.

Q: ping? this is not fully convincing…​ isn’t --socket-mem to allocate hugepage for vrouter (not to VM) only? or this is actually a same "global" system-wise parameter just as the kernel hugepagesz=1G hugepages=40 parameter?

A (LD): This is also something we have to explain clearly. Hugepages use in DPDK are really badly explained. And, once again this is not so complex as it seems.

You just have to keep in mind that :

- Packet are put into memory at one place and never copied

(only descriptors are moving from one Q to another)

- Consequently “memory” area where the packet are put must be shared between DPDK vrouter and all instances (what ever the instance is – DPDK or not DPDK)

Then, you have DPDK setup :

- At system level : to allocate a given amount of Memory as “hugepage type”

- At vrouter level: to use an amount of system hugepages to store packets (in mbufs)

- vRouter is using CPU on a same NUMA (at least this is the recommended setup if you want to avoid performance issues).

- Virtual instances are using CPU on both NUMA (and most probably on the other NUMA which is not used by vrouter – because you have lots of CPU on this second one available for your VMs)

So, in a short, you have instances running on both NUMA. They have to be able access packets that are referenced by descriptors (that vrouter as put in vNIC RX queue).

This is why, by default we spread hugepage memory allocation on both NUMA.

Here, is shown how huge pages are used.

So, first you are allocating HugePage at system level (at startup for 1G huge pages):

default_hugepagesz=1GB hugepagesz=1G hugepages=40 hugepagesz=2M hugepages=40

I guess, that Huge pages are equally balanced on both NUMA (to be checked)

Then you are requesting at vrtouter level a part of them for vrouter DPDK application need (to store both underlay and VM packets):

--socket-mem <value>,<value>

Q: ping? shouldn’t disabling HT archive max perf?

A (LD): HT matter is not a yes or no answer. It depends …

First, things to consider … HT has been created to improve hardware use efficiency. Idea is, we are creating “two virtual CPU” on a single Hardware CPU. It improve Physical CPU usage as the the second virtual CPU can use the physical resource when the first one is sleeping (like VM usage is improving Physical BM utilization)

As the main DPDK principle is to never let a CPU to sleep … we should answer no !!!

But, from the customer perspective which is using Bare Metal Compute to run VM, his answer is YES, I want it !!!

So, in general every customer is enabling HT on his servers. So, the question is … for vrouter needs, will we recommend to use only one “logical CPU” per physical CPU ?

Or do we recommend to use both (aka siblings) on physical CPU ?

Here, again the answer is not so clear, and can’t be Yes or No.

Refer to this section: Last step: sibling consideration

Two (simplified) situations:

- highest throughput is expected and number VM per compute is no the first criteria
 do not use siblings

- VM per compute is the first criteria. An average DPDK vrouter performance is largely enough
lets use siblings

Q: ping: talk about both algorithm.

A (LD)

First, we have to override this is RedHat specific. Ubuntu is using only one mechanism.

Second (not for the book), but this is not so clear on RedHat side. First idea was that new tuned partitioning will replace isolcpus historical mechanism.

But today this is not so clear on Redhat side, and they recommend to use both !!!.

Mainly because isolcpus is changing the scheduling algorithm from cooperative to preemptive, so it is more efficient for isolation.

This is why in my opinion we have to explain that:

- two mechanisms are proposed by redhat

- describe each one (how to configure it, what it does)

- explain we are recommending to use both of them to have the better CPU isolation as possible

Q: ping? explain why?

before Contrail 20.03 release, it is not recommended to use such an isolation method. If used some packet drops could randomly occur and vrouter performances are not stable.

A (LD): For a lot of complex reason hard to explain. One of them (easiest to understand) is:

because before 20.03 we were not offering an ability to pin correctly service threads.

when we were enforcing a too strict CPU isolation service threads were sometime using vrouter allocated polling and processing CPUs.

Since 20.03 we are proposing a setup in which:

- vrouter processing CPUs are assigned to well defined CPU list

- vrouter control and service CPUs are assigned to another well defined CPU list

OpenStack is providing a mechanism to assign a well defined CPU list to Virtual Instances

Consequently at system level we can enforce a strict isolation to remove vrouter and openstack assigned CPU to the "common" pool.

A small common pool is kept for Operating System usage (in a shart Hypervisor needs)

To be supplemented by Kiran and Premecz.

Q ping? should be = instead of :?

TRIPLEO_HEAT_TEMPLATE_KERNEL_ARGS: "isolcpus=7,9-35,43,45-71"

Because, this variable content will be added to GRUB “command line” which is using shell like syntax.

Q ping? this "vhost userspace process" is kernel vrouter only?

A (LD):

In my opinion we have to explain:

- DPDK vrouter is running in user space (this is the goal of DPDK)

- Kernel vouter dataplane is running in Kernel space (vrouter.ko module)

But afer, this kernel/user space has not only to be considered at vrouter level.

You aslo have to consider the virtualization layer:

- what about virtio (QEMU capaability) ?

- what about the VM itself (using DPDK or not) ?

QEMU, few years ago was not able to run in userspace. So, it was creating performance issue when used with DPDK applications.

VNF, were also running Kernel packet processing stack

Drawback is interupt mechanisms it required that is slowering packet processing and Kernel to User constext swap (which is CPU intensive) is requires.

Nowaday, lots of VNF are DPDK and QEMU is supporting lots of scenarios.

Ideal situation is to get everything running at userspace level:

- DPDK vrouter

- DPDK VNF (multiQ - same number of Q than vrouter)

- virtio also running in user space (vhost-user)

All these situations are described in this diagram:

The last drawing at right is the best situation and the one that is providing the best performance.

The wort is the first diagram at left. Both diagrams in the middle are providing drawbacks:

- When DPDK VM is used onto a Kernel vRouter, vrouter will will slow down packet processing

- When Kernel VM is used onto a DPDK vrouter, Kernel VM will slow down packet processing

So, this is a pity situation to mix DPDK and not-DPDK element (but sometimes it can’t be avoided)

So, we have to explain that vrouter can’t solve everything. Part of the job has to be done at VNF layer.

In my opinion, the main goal of this book is to explain the (nowadays) optimal situation:

- DPDK vrouter

-  UDP overlay protocol (MPLSoUDP or VxLAN)

- 200X release: with 2 hash mechanism disabled (discuss with Kiran and Premecz about X)

- DPDK VNF (multiQ – with same number of Q than vrouter)

Q: ping? interface type should be vhostuser?

A (LD): This is something that is not fully clear for me. When is use OVS DPDK I clearly see that VM interfaces are vhost user. To be clarified with Kiran and Premecz.

Q: ping? this is great diagram showing overall allocations. the numbering seems not correct. see hwloc-ls GUI and lscpu

A (LD): the problem is that we are mixing platforms ...

All examples I've added in this document are comming from Orange which is using to kinds of plaforms (72 lcores or 48 lcores).

So, I'm trying to be consistent in perf guide. But some examples are provided from other platforms. So, it can lead to confusion.

CPU numbering is not always following same rules:

- some suppliers are using odd and even numbers to differentiate Numa0 from Numa1 CPUs

- others are using first half and second half numbers to differentiate Numa0 from Numa1 CPUs

This is bringing confusion ...

Here an example - 48 lcores:

# lscpu | grep NUMA

NUMA node(s):          2

NUMA node0 CPU(s):     0-11,24-35

NUMA node1 CPU(s):     12-23,36-47

Physical Cores are ID 0 to 23

First half = Numa0

Second half = Numa1

Siblings are ID 24 to 27

Here is another example 72 lcores (like Orange Model):

Physical Cores are ID 0 to 35

Even = Numa0

Odd number = Numa1

Siblings are ID 36 to 71

NUMA node0 CPU(s):

PHY cores: *0 2 4 6 8 10 12 14 16 18 20 22 24 26 28 30 32 34*

HT cores : *36 38 40 42 44 46 48 50 52 54 56 58 60 62 64 66 68 70*

NUMA node1 CPU(s):

PHY cores: *1 3 5 7 9 11 13 15 17 19 21 23 25 27 29 31 33 35*

HT cores : *37 39 41 43 45 47 49 51 53 55 57 59 61 63 65 67 69 71*

Updated situation with 48 CPUs:

NUMA node0 CPU(s):

PHY cores: *0 2 4 6 8 10 12 14 16 18 20 22*

HT cores : *24 … 46*

NUMA node1 CPU(s):

PHY cores: *1 3 5 7 9 11 13 15 17 19 21 23*

HT cores : *25 …. 47*

And the diagram I’ve provided in the book:

I can provide an updated diagram. But pay attention all netmask I’ve provided are following this numbering rule:

0x154 …. Has to be changed everywhere if we are changing CPU model

Q: ping? statement #1 and #2 above is conflicting.

1: When Hyper Threading is used, we will book both physical CPUs and their sibling ones for DPDK vRouter packet processing threads.

2: vRouter packet processing threads will be assigned with physical CPUs in same NUMA. Sibling CPUs are unused

A (LD): yes, your right.

To be updated (last part of second sentence to be removed):

2: vRouter packet processing threads will be assigned with physical CPUs in same NUMA.

Q: ping? confusing. text makes sense, but not corelating to the diagram. 2p2ht means 2 pcores, both enabled ht⇒4lcores in use?

A (LD):

It is consistent. Let see the situation with 4 lcores usage:

2 situations:

2p + 2h   --> diagram = 3 MPPS

4p + 0h   --> diagram = 5.2 MPPS

3 / 4 = 0.75 (almost 0.8) ==> 1,6 MPPS per Physical CPU

5.2 / 4 = 1.3             ==> 1,3 MPPS per Physical CPU (near to 1,25)

Q: ping? didn’t say why..

Large pages are required for each instance (even any non-DPDK instance) running on hosts with DPDK vrouter. If large pages are not present in the guest, the interface will appear but will not function.

A (LD): Indeed. See earlier explanations. Second sentence is explaining why.

You just have to keep in mind that :

- Packet are put into memory at one place and never copied (only descriptors are moving from one Q to another)

- Consequently “memory” area where the packets are put must be shared between DPDK vrouter and all instances (whatever the instance is – DPDK or not DPDK)

Q: ping? since 4.0?

Since Contrail 5.0 release and later, Contrail is containerized. Kernel vrouter module is still using /etc/modprobe.d/vrouter.conf file to get specific dimensioning values.

A (LD): I do not know I do not have any 4.0 lab. I guess there is no difference (as we are telling “is still using”). But, it is not so important in my opinion. Let's focus now on 19.X for this book. We can look for this information for the current (and internal only) perf guide document.

Q ping: great visualization! maybe worth a better quality image here

A (LD): as now we are using .doc word format, it will be easier for me to integrate better quality images. Let’s see how we will integrate images in the definitive document.